For your use case, I would model it as end-of-life emissions for physical assets/furniture.

1. Basic formula

For a chair:

EOL CO₂e=Weight (kg)×Treatment %×Emission Factor (kg CO₂e/kg)

For example, if a 20 kg chair is:

- 70% recycled

- 20% sent to landfill

- 10% incinerated

then:

CO2e=20×70%×EFrecycling+20×20%×EFlandfill+20×10%×EFincineration

2. Furniture should ideally be broken into materials

This is particularly useful for your chairs/tables/furniture asset-health/compliance model.

For example, a chair could contain:

| Material | Weight | EOL treatment |
| --- | --- | --- |
| Steel | 8 kg | Recycling |
| Plastic | 5 kg | Recycling |
| Foam | 3 kg | Landfill |
| Fabric | 2 kg | Recycling |
| Wood | 2 kg | Reuse/recycling |

Then:

Total EOL CO2e=(Material Weight×Treatment Rate×EF

This gives you a much more accurate asset-level calculation.

3. But there is an important distinction for unused furniture

If the furniture is still usable, don't automatically classify it as "recycling."

You could have an End-of-Life hierarchy such as:

Reuse → Refurbish → Resell/Donate → Recycle → Incinerate → Landfill

For example:

100 chairs → 60 reused + 25 refurbished + 10 recycled + 5 landfill

Your system could calculate the emissions associated with each pathway.

4. Scope 2 vs Scope 3

For your scenario, if an external recycling/waste-management company collects and processes the furniture:

Your organization's recycling/disposal activity would generally be accounted for within Scope 3, depending on the specific organizational/accounting boundary and category applicable to the activity.

The electricity consumed by the recycling facility itself is potentially Scope 2 for that recycling facility, not automatically Scope 2 for your organization.

5. Recommended formula for your project

I would therefore use:

Furniture EOL CO2e=mWm×Pm×EFm,t+Transport CO2e

Where:

- Wm= weight of material

- Pm= percentage going through treatment t

- EFm,t= emission factor for that material + treatment

- Transport CO₂e = transportation emissions, where included in your chosen boundary

Example asset record:

Asset Type       = Office Chair

Asset Weight     = 20 kg

Material         = Steel + Plastic + Foam + Fabric

Condition        = Unused

EOL Decision     = Recycle

Recycling Rate   = 85%

Transport        = 25 km

EOL CO2e         = calculated

For your Sustainability Insights concept, I'd actually recommend keeping "Reuse CO₂e", "Refurbishment CO₂e", "Recycling CO₂e", "Landfill CO₂e" and "Avoided/beneficial emissions" as separate metrics, rather than putting everything into one CO₂e number. This will make the asset-level sustainability dashboard considerably more useful.